Manual Install Instructions for
HDTV998E PCIe Card

- Shut down the Reflect player.

- Copy or overwrite these files in the ReflectPlayer directory:

application/x86/Board1089.dll

application/x86/GenericDevices.dll

application/x86/PlxApi650.dll

- Start ReflectPlayer

- Install the drivers for the 1121 board:
(For a 64-bit OS architecture is x64, for a 32-bit OS architecture is x86.)

driver/architecture/hdtv998e.inf – When Windows detects a new card point it to this file

driver/architecture/hdtv998e.cat

driver/architecture/hdtv998e.sys

driver/architecture/WdfCoInstaller01009.dll

- Configure the ReflectPlayer to use the “Sencore” Plugin

- Reboot